Software Release Notes

This is v1.0 of the Solar Controller software and implements all the features mentioned in the Article except:-

Known issues

1) The article stated: “A warning message is displayed rather than the total if it has not collected enough readings since the last reset.” This feature is not yet implemented.

2) Part 1 stated that the bottom sensor was used for Legionella prevention heating. This software version uses the middle sensor instead as, when hot water is drawn off, the bottom sensor cools down as cold water replaces the water drawn off. Thus a lot of heat would have to be added to the tank to achieve 60C at the bottom sensor, presenting a scalding risk.

Notes

1) Please report any bugs found or suggestions for improvements through a private message to Edward Chase (username echase) via EPE’s Chatzone. http://www.chatzones.co.uk

2) On the maintenance display the line under the panel temperature reads:-

Non adjusted time, Sens1_error_count, Pump state

Where Sens1_error_count is the number of times in one day the panel sensor has exceeded Oheat_temp or Open_circuit_value or gone under Short_circuit_value.

The line under the bottom temperature reads:-

Dawn time, Dusk time, Pump state

The line under the aux temperature reads:-

Backup heat relay on (1= is on now), Backup heat flag (1= has heated today), Pump state